Metadata for the Great Lakes and St. Lawrence River

Medium Resolution Vector Shoreline Data

Identification_Information:

Citation:

Originator: Great Lakes Environmental Research Laboratory (compiler)

Publication_Date: 19971201

Publication_Time: 00000000

Title: Great Lakes and St. Lawrence River Medium Resolution Vector Shoreline Data

Edition: 1.0

Geospatial_Data_Presentation_Form: Map

Publication_Information:

Publication_Place: Ann Arbor, MI, U.S.A.

Publisher: Great Lakes Environmental Research Laboratory

Other_Citation_Details: NOAA Technical Memorandum ERL-GLERL xx

Online_Linkage: ftp://ftp.glerl.noaa.gov/gis/shoreline/

Larger_Work_Citation: Vector Coastline Data for the Conterminous U.S. including

the Great Lakes, National Geophysical Data Center, Boulder, CO, USA.

Scale_Denominator: varies from 24000 to 250000

Description:

Abstract:  A medium resolution vector map of the Canadian and U.S. Great Lakes and St. Lawrence River shoreline is segmented into reaches and classified using a three tier scheme representing the geomorphic nature of the shoreline, the extent of shoreline protection, and the geomorphic nature of the nearshore subaqueous shoreline.  The classification scheme is as follows:

Shoreline Geomorphic Nature Classification:

1    High (>15m) Bluff

2    High (>15m) Bluff with Beach

3    Low (<15m) Bluff

4    Low (<15m) Bluff with Beach

5    Sandy/Silty Banks

6    Clay Banks

7    Sandy Beach/Dunes

8    Coarse Beaches

9    Baymouth-Barrier Beaches

10   Bedrock (Resistant)

11   Bedrock (Non-resistant)

12   Low Riverine/Coastal Plain

13   Open Shoreline Wetlands

14   Semi-Protected Wetlands

15   Composite

16   U.S. Shore:  Unclassified

16   Canadian Shore: Artificial

17   U.S. Shore: Artificial

17   Canadian Shore: Unclassified

99   Unclassified (coded by the compiler)

Shoreline Protection Classification:

1    Highly Protected: 70-100 percent of reach/segment protected

2    Moderately Protected: 40-70 percent of reach/segment protected

3    Minor Protection: 15-40 percent of reach/segment protected

4    No Protection: <15 percent of reach/segment is protected

5    Non-Structural

6    Unclassified

9    Unclassified (coded by the compiler)

Nearshore Subaqueous Geomorphic Nature Classification:

1    Clay

2    Sand

3    Sand/Gravel Lag Over Clay

4    Bedrock (Resistant)

5    Bedrock (Non-Resistant)

6    Unclassified

9    Unclassified (coded by the compiler)

Combined Classification:

The above codes are appended into a 3 to 4 digit code representing the combined classification. The right-most digit represents the nearshore subaqueous classification, the second digit from the right represents the shoreline protection classification, and the left-most one or two digits represent the shoreline geomorphic classification.

Purpose:

The shoreline vectors and classification scheme were originally developed by Environment Canada and the U.S. Army Corps of Engineers to evaluate the influence of lake level effects on erosion for the International Joint Commissions Levels Reference Study.   Potential exists for use of the data in shoreline management, and environmental and coastal processes studies.

Supplemental_Information:

Revisions:  None

Reviews_Applied_to_Data:

The U.S. data were received from the U.S. Army Corps of Engineers, Detroit District.  After conversion by GLERL of the data from INTERGRAPH vector format to the ARC/INFO vector format (see Procedures Used), the vector data were checked to ensure consistent topology. Visual inspection of the processed data overlaying the original data and other shoreline datasets was used to check for any gross processing errors. Shoreline classification codes in the processed U.S. data were verified manually by comparing computer and hard copy maps to hard copy maps provided by the Detroit District.  Reviews applied during the production of the data by the originators are described in the references listed below (see Other References Cited).

The Canadian data was received from the Water Issues Division of Environment Canada, Ontario Region, in Arc/Info vector format.  After modifying attribute item definitions and appending the attribute information to the vector files (see Procedures Used), the processed data were visually inspected as described above and the classification codes were verified by comparing computer and hard copy maps to hard copy maps provided by Environment Canada.  Reviews applied during the production of the data by the originators are described in the references cited below (see Other References Cited).

Related_Spatial_and_Tabular_Data_Sets:

None.

Other_References_Cited:

Working Committee 2, 1993.  Annex 2, Land Use and Management. International Joint Commission Levels Reference Study, Great Lakes-St. Lawrence River Basin. International Joint Commission, Washington, D.C. and Ottawa, Canada.

United States Army Corps of Engineers, 1992.  Summary, Shore Mapping and Classification Project: United States Shoreline.  Memorandum for Record, Coastal Engineering Research Center, Vicksburg, Mississippi, 14 pp.

Geomatics International, 1992.  Great Lakes Shoreline Classification and Mapping Study:     Canadian Side.  Final Report Submitted to the Erosion Processes Task Group, Working     Committee 2, International Joint Commission Levels Reference Study, International Joint      Commission, Washington, D.C. and Ottawa Canada, 44 pp. plus Appendices and Maps.

Time_Period_of_Content:

Calendar_Date: Unknown

Currentness_Reference:

The aerial photography used to generate the U.S. shoreline vector data was acquired in 1979 for the State of Michigan and in 1987-1989 for the other seven Great Lakes.   The time period of content for the Canadian shoreline vector data is unknown to the compiler.

The U.S. and Canadian shoreline classification was completed in 1992 from a variety of variously dated materials.  The shoreline protection classification may change significantly as time increases from the date of the original classification.  More information on the currentness of the vector and classification data is contained in the references cited above (see Other References Cited).

Access_Constraints:

There are no access constraints on the U.S. data.  Unlimited use and reference of the Canadian shoreline data is granted upon condition that Water Issues Division of Environment Canada - Ontario Region is credited as the source.

Data_Set_Credit:

The data were originally produced by the U.S. Army Corps of Engineers and Environment Canada for the International Joint Commission's Levels Reference Study, Great Lakes-St. Lawrence River Basin, by Working Committee 2, Land Use and Management, and used to assess the influence of lake levels on shore erosion.  The U.S. shoreline data were obtained from the Detroit District, U.S. Army Corps of Engineers, Detroit, Michigan.  The Canadian shoreline data were obtained from Water Issues Division of Environment Canada - Ontario Region.  The Great Lakes Environmental Research Laboratory, Ann Arbor, Michigan compiled the data and translated them into multiple common formats including the Topological Vector Profile of the Spatial Data Transfer Standard, to enhance the accessibility of the data.  This work was done in conjunction with the National Geophysical Data Center to develop medium resolution coastline data for the conterminous United States, and was funded by NOAAs Earth System Data and Information Management Program.

Completeness_Report:

The data include vector representations of the Great Lakes shoreline, their connecting channels, the St. Lawrence River, and many (but not all) islands.  Some man-made features such as the Keweenaw Waterway (Lake Superior) are excluded.  The vector end-nodes of some of the U.S. counties and Canadian regions are not consistent with their adjacent county or region.  Most of the shoreline is classified with the exception of most islands, Isle Royale (Lake Superior) being a notable exception.

Cloud_Cover:

Not applicable.

Status:

Progress:  Complete

Maintenance_and_Update_Frequency: None planned

Spatial_Domain:

Bounding_Coordinates:

West_Bounding_Coordinate: -92.42496472 decimal degrees

East_Bounding_Coordinate: -71.52364405 decimal degrees

North_Bounding_Coordinate: 49.18507357 decimal degrees

South_Bounding_Coordinate: 39.98076086 decimal degrees

Keywords:

Theme:

Theme_Keyword_Thesaurus:  None

Theme_Keyword:  Great Lakes, shoreline, coastline, geomorphic classification, shore protection classification

Place:

Place_Keyword_Thesaurus:  None

Place_Keyword:  Great Lakes and St. Lawrence River

Stratum:

Stratum_Keyword_Thesaurus:  None

Stratum_Keyword: Shoreline

Temporal:

Temporal_Keyword_Thesaurus:  None

Temporal_Keyword: Contemporary

Use_Constraints:

Vector Shoreline:

The vector data are intended for general planning, study, and illustrative purposes.  The data are not intended for uses that require a high degree of positional accuracy.

Shoreline classification:

The process of shoreline classification was essentially subjective and based almost entirely upon descriptive criteria. The classification of each reach was the result of the subjective interpretation of the individual doing the work and was based on their understanding of the classification scheme, the information available, and the experience of the individual. The shoreline protection classification may change significantly as time increases from the date of the original classification. Other limitations are the inadequacy of the classification scheme to encompass the complete range of shoreline characteristics, and insufficient detail of the data sources to permit classification. Inconsistencies in classification may exist.

Procedures_Used:

U.S. Data:

INTERGRAPH format shoreline files, by U.S. county, were obtained from the Detroit District U.S. Army Corps of Engineers ftp server, ftp://ftp@155.79.125.194/pub/.  These files were stored on the HP 735/9000 GIS workstation at the Great Lakes Environmental Research Laboratory. The procedures used to convert and process a given INTERGRAPH file were essentially the same for all of the counties: a file was converted to two ARC/INFO vector format covers--one for the shorelines and one for the breaklines (hash marks that intersect the shorelines at the reach endpoints)--using the ARC command IGDSARC with the subcommand shoreline 61 * * * * for shorelines and break 3 * * * * for breaklines.  ARC attribute tables (AAT), in INFO format, were then generated for the shoreline and breakline covers using the ARC command BUILD with the line option.  Next, the shoreline and breakline covers that were generated from the same INTERGRAPH file were appended together using the ARC command APPEND.  An AAT was then built for the resulting appended cover using BUILD with the line option.

Canadian Data:

The Canadian data were obtained from Waters Issue Division, Environment Canada in ARC/INFO format.  Some of these covers had items which were dropped from the covers' associated arc attribute tables.  These items included PLOTKEY, REACHKEY, ITEM2, and ITEM1.  This was done to prepare the AAT files before joining the INFO tables that contained the shoreline classification codes.  The items in these tables included REACH_KEY, GEOMORF, PROTECTION, and NEARSHORE.  The item GEOMORF was altered to spell GEOMORPH.  Then these tables were joined to each shoreline cover's AAT using the ARC/INFO command, JOINITEM, specifying the cover-id item in the AAT as the relate item and using the LINK option. This process placed the Canadian data in the same format as the U.S. shoreline data.

U.S. and Canadian Data:

The covers were projected from their original projections and map coordinate systems to Albers Equal-Area Conic.  This was done by using the ARC command PROJECT with the cover option and the appropriate input and output parameters, listed below. Note that for Michigan county shoreline covers, which were in the Michigan State Plane Coordinate System prior to projecting to Albers, ARC software permitted specifying "stateplane" for the projection, instead of specifying an actual projection and projection parameters.  When stateplane is entered, the  user is prompted for a FIPS code zone number (FIPSZONE).  FIPSZONE numbers were determined from Table 4 in the ARC command reference for the PROJECT command.  The FIPSZONES entered were 2111, 2112, and 2113 (North, Central, and Southern Michigan, respectively).

---Input (source) Projection Parameters Used---

For southern Michigan counties:

Projection: Lambert Conformal Conic

Units: Feet

FIPS code zone number: 2113

Datum: NAD27

Spheroid: Clarke 1866

1st Standard Parallel: 42 06 00  (degrees, minutes, seconds)

2nd Standard Parallel: 43 40 00

Central Meridian: -84 20 00

Origin (Latitude): 41 30 00

False Easting (m): 609,601.21920

False Northing (m): 0

For central Michigan counties:

Projection: Lambert Conformal Conic

Units: Feet

FIPS code zone number: 2112

Datum: NAD27

Spheroid: Clarke 1866

1st Standard Parallel: 44 11 00  (degrees, minutes, seconds)

2nd Standard Parallel: 45 42 00

Central Meridian: -84 20 00

Origin (Latitude): 43 19 00

False Easting (m): 609,601.21920

False Northing (m): 0

For northern Michigan counties:

Projection: Lambert Conformal Conic

Units: Feet

FIPS code zone number: 2111

Datum: NAD27

Spheroid: Clarke 1866

1st Standard Parallel: 45 29 00  (degrees, minutes, seconds)

2nd Standard Parallel: 47 05 00

Central Meridian: -87 00 00

Origin (Latitude): 44 47 00

False Easting (m): 609,601.21920

False Northing (m): 0

For counties in Wisconsin, Illinois, and Indiana:

Projection: Oblique Mercator

Units: meters

Spheroid: Clarke 1866

Projection Type: 2 (central line defined by one point and its angle of azimuth)

Scale factor at the projection center: 0.9999

Longitude of the projection center: -87.0

Latitude of the projection center: 44.0

Azimuth at the projection center: 15

False easting: -1000000.0

False northing: -4300000.0

For counties in Minnesota:

Projection: Oblique Mercator

Units: meters

Spheroid: Clarke 1866

Projection Type: 2 (central line defined by one point and its angle of azimuth)

Scale factor at the projection center: 0.9999

Longitude of the projection center: -88 50 00.256

Latitude of the projection center: 47 12 21.554

Azimuth at the projection center: 285.6952

False easting: 9000000.0

False northing: -1600000.0

For counties in Ohio and New York:

Projection: Oblique Mercator

Units: meters

Spheroid: Clarke 1866

Projection Type: 2 (central line defined by one point and its angle of  azimuth)

Scale factor at the projection center: 0.9999

Longitude of the projection center: -78.0

Latitude of the projection center: 44.0

Azimuth at the projection center: 55.666670

False easting: -3950000.0

False northing: -3430000.0

All Canadian Shorelines:

Projection: Universal Transverse Mercator (UTM)

Units: Meters

UTM Zones:

Zone 16 for Canadian Shorelines of Lake Superior and St. Mary's River

Zone 17 for Canadian Shorelines of Lake Huron, St. Clair River, Lake St. Clair,  Detroit River, Lake Erie, Western Lake Ontario, and the Niagara River

Zone 18 for Canadian Shorelines of Eastern Lake Ontario and the St. Lawrence River

--Output Projection Parameters Used--

Projection: Albers Equal-Area Conic

Units: meters

Spheroid: Clarke 1866

1st Standard Parallel: 29.5

2nd Standard Parallel: 45.5

Central Meridian: -96

Origin (Latitude): 23.0

False Easting (m): 0

False Northing (m): 0

U.S. Data:

The U.S. covers, when first converted from INTERGRAPH to ARC, had pseudonodes at every vertex.  Thus, arc segments were very short and did not extend across the reach from one breakline to another.  After projection to Albers Equal Area, the ARC command CLEAN was used to establish arc-node topology so that pseudo-nodes could be removed.  The covers were processed using CLEAN with the line option, a dangle length of 0, and a fuzzy tolerance of 2 meters.  The coordinate precision of the U.S. covers remained double precision.  All covers were CLEANed only once.  All pseudo-nodes were then removed by selecting all arcs using the ARC module ARCEDIT and then using the ARCEDIT command UNSPLIT with the none option.  Changes to the edit coverage were saved and ARCEDIT was quit.

AATs were rebuilt using BUILD with the line option. Then ARCEDIT was again launched and a coverage chosen for editing.  All arcs were selected, and the user-id (<cover>-id) was reset to equal the  internal-id (<cover>#) using the ARCEDIT command CALCULATE.  Then the items GEOMORPH, PROTECTION, NEARSHORE, and COMBINED were added to the AAT using the ARCEDIT command ADDITEM with input and output width of 4 and data type integer.

A graphical user interface was created to simplify the editing and coding of arcs.  Arcs were selected and edited using ARCEDIT commands such as DELETE and UNSPLIT and coded using CALCULATE.  Codes for the COMBINED item were determined from hardcopy maps generated from the original INTERGRAPH files, obtained from the Detroit District U.S. Army Corps of Engineers.  An algorithm was developed to parse the COMBINED code into GEOMORPH, PROTECTION, and NEARSHORE values and automatically update these item values whenever a COMBINED code was entered for a selected arc.  Arcs whose codes were ambiguous were given a combined value of 9999, which was also parsed, giving 99, 9,  and 9 for the three classifications.  All arcs representing breaklines were deleted, leaving only pseudonodes to mark the point along the shoreline where reaches end.  In some cases, pseudo-nodes could not be removed (because ARC limits the number of vertices in an arc to 500). These pseudonodes can be distinguished from breakpoints because the

codes of the adjoining arcs are the same.

U.S. and Canadian Data:

All covers in the Albers Equal Area projection were converted to ARC/INFO export files using the ARC command EXPORT with the COVER and FULL options.  The covers were also converted to ASCII generate format files using the ARC command UNGENERATE with the LINE option.  The companion INFO AAT tables were converted to comma delimited ASCII files using the ARC TABLES command UNLOAD with the DELIMITED option.  The covers were also projected and the same process used to create ASCII UNGENERATE files in geographic coordinates. To build polygon topology for conversion to SDTS,  the covers projected to geographic coordinates were processed using the ARC command BUILD with the POLY option.  These covers were then converted to SDTS Vector Topological Profile files using the ARC command SDTSEXPORT.

Point_of_Contact:

Contact_Information:

Contact_Person_Primary:

Contact_Person: Deborah H. Lee

Contact_Organization: Great Lakes Environmental Research Laboratory

Contact_Position: Hydrologist

Contact_Address:

Address_Type: mailing and physical address

Address: 2205 Commonwealth Blvd.

City: Ann Arbor

State_or_Province: MI

Postal_Code: 48105-1593

Country: U.S.A.

Contact_Voice_Telephone: 313-741-2148

Contact_Facsimile_Telephone: 313-741-2055

Contact_Electronic_Mail_Address: lee@glerl.noaa.gov

Hours_of_Service: 8:00-4:30 Eastern

Security_Information:

Security_Classification_System: None

Security_Classification:  Unclassified

Security_Handling_Description: None

Native_Data_Set_Environment: HP-UX UNIX, ARC/INFO version 7.0.4

Data_Quality_Information:

Attribute_Accuracy:

Attribute_Accuracy_Report:  See Entity_Attribute_Information

Quantitative_Attribute_Accuracy_Assessment:

Attribute_Accuracy_Value:  See Explanation

Attribute_Accuracy_Explanation:  Attribute accuracy is described, where present, with each attribute defined in the Entity and Attribute Section.

Logical_Consistency_Report:  Chain-node topology present.

Positional_Accuracy:

Horizontal_Positional_Accuracy:

Horizontal_Positional_Accuracy_Report: The horizontal positional accuracy has not been determined.

Vertical_Positional_Accuracy:

Vertical_Positional_Accuracy_Report: N/A

Spatial_Data_Organization_Information:

Direct_Spatial_Reference_Method:  Vector

Point_and_Vector_Object_Information:

SDTS_Terms_Description:

SDTS_Point_and_Vector_Object_Type:  Point

Point_and_Vector_Object_Count:  0

SDTS_Point_and_Vector_Object_Type:  String

Point_and_Vector_Object_Count: varies

SDTS_Point_and_Vector_Object_Type:  GT-polygon composed of chains

Point_and_Vector_Object_Count:  0

Spatial_Reference_Information:

Horizontal_Coordinate_System_Definition:

Planar

Map_Projection:

Map_Projection_Name:  ALBERS

Longitude_of_Central_Meridian:  -96 decimal degrees

Latitude_of_Projection_Origin:  23 decimal degrees

Latitude_of_First_Standard_Parallel:  29.5 decimal degrees

Latitude_of_Second_Standard_Parallel:  45.5 decimal degrees

False_Easting:  0.00000

False_Northing:  0.00000

Geodetic Model

Horizontal_Datum_Name:

Ellipsoid_Name:  Clarke 1866

Semi-major_Axis: 6,378,206.4

Denominator_of_Flattening: 294.98

Entity_and_Attribute_Information:

Detailed_Description:

Entity_Type:

Entity_Type_Label: <cover>.AAT

Entity_Type_Definition:  Arc Attribute Table

Entity_Type_Definition_Source:  ESRI, Inc.

Attribute:

Attribute_Label:  FNODE#

Attribute_Definition:  Internal number of from-node

Attribute_Definition_Source:  Computed

Attribute_Domain_Values:

Enumerated_Domain:

Enumerated_Domain_Value:  Sequential unique positive integer

Attribute:

Attribute_Label:  TNODE#

Attribute_Definition:  Internal number of to-node

Attribute_Definition_Source:  Computed

Attribute_Domain_Values:

Enumerated_Domain:

Enumerated_Domain_Value:  Sequential unique positive integer

Attribute:

Attribute_Label:  LPOLY#

Attribute_Definition:  Internal number of poly to left of arc

Attribute_Definition_Source:  Computed

Attribute_Domain_Values:

Enumerated_Domain:

Enumerated_Domain_Value:  Sequential unique positive integer

Attribute:

Attribute_Label:  RPOLY#

Attribute_Definition:  Internal number of poly to right of arc

Attribute_Definition_Source:  Computed

Attribute_Domain_Values:

Enumerated_Domain:

Enumerated_Domain_Value:  Sequential unique positive integer

Attribute:

Attribute_Label:  LENGTH

Attribute_Definition:  Length of arc in coverage units

Attribute_Definition_Source:  Computed

Attribute_Domain_Values:

Enumerated_Domain:

Enumerated_Domain_Value:  Positive real numbers

Attribute:

Attribute_Label: <cover>#

Attribute_Definition:  Internal feature number

Attribute_Definition_Source:  Computed

Attribute_Domain_Values:

Enumerated_Domain:

Enumerated_Domain_Value:  Sequential unique positive integer

Attribute:

Attribute_Label: <cover>-ID

Attribute_Definition:  User-assigned feature number

Attribute_Definition_Source:  User-defined

Attribute_Domain_Values:

Enumerated_Domain:

Enumerated_Domain_Value:  Integer

Attribute:

Attribute_Label:  GEOMORPH

Attribute_Definition:  Geomorphic Class

Attribute_Definition_Source:  Working Committee 2, 1993. Annex 2, Land Use and Mgt., Levels Reference Study.

Attribute_Domain_Values:

Enumerated_Domain:

Enumerated_Domain_Value: 1

Enumerated_Domain_Value_Definition: High (>15m) Bluff

Enumerated_Domain_Value: 2

Enumerated_Domain_Value_Definition: High (>15m) Bluff with Beach

Enumerated_Domain_Value: 3

Enumerated_Domain_Value_Definition: Low (<15m) Bluff

Enumerated_Domain_Value: 4

Enumerated_Domain_Value_Definition: Low (<15m) Bluff with Beach

Enumerated_Domain_Value: 5

Enumerated_Domain_Value_Definition: Sandy/Silty Banks

Enumerated_Domain_Value: 6

Enumerated_Domain_Value_Definition: Clay Banks

Enumerated_Domain_Value: 7

Enumerated_Domain_Value_Definition: Sandy Beach/Dunes

Enumerated_Domain_Value: 8

Enumerated_Domain_Value_Definition: Coarse Beaches

Enumerated_Domain_Value: 9

Enumerated_Domain_Value_Definition: Baymouth-Barrier Beaches

Enumerated_Domain_Value: 10

Enumerated_Domain_Value_Definition: Bedrock (Resistant)

Enumerated_Domain_Value: 11

Enumerated_Domain_Value_Definition: Bedrock (Non-resistant)

Enumerated_Domain_Value: 12

Enumerated_Domain_Value_Definition: Low Riverine/Coastal Plain

Enumerated_Domain_Value: 13

Enumerated_Domain_Value_Definition: Open Shoreline Wetlands

Enumerated_Domain_Value: 14

Enumerated_Domain_Value_Definition: Semi-Protected Wetlands

Enumerated_Domain_Value: 15

Enumerated_Domain_Value_Definition: Composite

Enumerated_Domain_Value: 16

Enumerated_Domain_Value_Definition: US: Unclassified, Canada: Artificial

Enumerated_Domain_Value: 17

Enumerated_Domain_Value_Definition: US: Artificial, Canada: Unclassified

Enumerated_Domain_Value: 99

Enumerated_Domain_Value_Definition: Unclassified (coded by compiler)

Attribute:

Attribute_Label:  PROTECTION

Attribute_Definition:  Shoreline Protection Classification

Attribute_Definition_Source:  Working Committee 2, 1993. Annex 2, Land Use and Mgt., Levels Reference Study.

Attribute_Domain_Values:

Enumerated_Domain:

Enumerated_Domain_Value: 1

Enumerated_Domain_Value_Definition: Highly Protected: 70-100 percent of        reach/segment protected

Enumerated_Domain_Value: 2

Enumerated_Domain_Value_Definition: Moderately Protected: 40-70 percent of reach/segment protected

Enumerated_Domain_Value: 3

Enumerated_Domain_Value_Definition: Minor Protection: 15-40 percent of reach/segment protected

Enumerated_Domain_Value: 4

Enumerated_Domain_Value_Definition: No Protection: <15 percent of reach/segment is protected

Enumerated_Domain_Value: 5

Enumerated_Domain_Value_Definition: Non-Structural

Enumerated_Domain_Value: 6

Enumerated_Domain_Value_Definition: Unclassified

Enumerated_Domain_Value: 9

Enumerated_Domain_Value_Definition: Unclassified (coded by compiler)

Attribute:

Attribute_Label:  NEARSHORE

Attribute_Definition:  Nearshore Soil or Geologic Parent Material

Attribute_Definition_Source:  Working Committee 2, 1993. Annex 2, Land Use and Mgt., Levels Reference Study.

Attribute_Domain_Values:

Enumerated_Domain:

Enumerated_Domain_Value: 1

Enumerated_Domain_Value_Definition: Clay

Enumerated_Domain_Value: 2

Enumerated_Domain_Value_Definition: Sand

Enumerated_Domain_Value: 3

Enumerated_Domain_Value_Definition: Sand/Gravel Lag Over Clay

Enumerated_Domain_Value: 4

Enumerated_Domain_Value_Definition: Bedrock (Resistant)

Enumerated_Domain_Value: 5

Enumerated_Domain_Value_Definition: Bedrock (Non-Resistant)

Enumerated_Domain_Value: 6

Enumerated_Domain_Value_Definition: Unclassified

Enumerated_Domain_Value: 9

Enumerated_Domain_Value_Definition: Unclassified (coded by compiler)

Attribute:

Attribute_Label:  COMBINED

Attribute_Definition:  3 to 4 digit code of appended Geomorph, Protection, and Nearshore Codes

Attribute_Definition_Source:  Working Committee 2, 1993. Annex 2, Land Use and Mgt. Levels Reference Study.

Overview_Description:

Entity_and_Attribute_Overview:

Entity_and_Attribute_Detail_Citation: Not Available

Distribution_Information:

Distributor:

Contact_Information:

Contact_Person_Primary:

Contact_Person: Deborah H. Lee

Contact_Organization: Great Lakes Environmental Research Laboratory

Contact_Position: Hydrologist

Contact_Address:

Address_Type: mailing and physical address

Address: 2205 Commonwealth Blvd.

City: Ann Arbor

State_or_Province: MI

Postal_Code: 48105-1593

Country: U.S.A.

Contact_Voice_Telephone: 313-741-2148

Contact_Facsimile_Telephone: 313-741-2055

Contact_Electronic_Mail_Address: lee@glerl.noaa.gov

Hours_of_Service: 8:00-4:30 Eastern

Resource_Description: Great Lakes and St. Lawrence River Medium Resolution Vector Shoreline Data

Distribution_Liability:

DISCLAIMER:

The coastline data contained in this publication were generated for use by NOAA's Great Lakes Environmental Research Laboratory (GLERL).  Although GLERL is making these data available to others who may find them of value, GLERL does not warrant, endorse, or recommend the use of these data for any given purpose. In no event will GLERL be liable to you or any third party for any direct, indirect, incidental, consequential, special, or exemplary damages or lost profits resulting from any use or misuse of these data.

Standard_Order_Process:

Digital_Form:

Digital_Transfer_Information:

Format_Name: SDTS Spatial Data Transfer Standards (FIPS 173)

Format_Name: ARC/INFO Export format (Albers Equal Area Projection)

Format_Name: ARC/INFO Generate format (Albers Equal Area Projection)

Format_Name: ARC/INFO Generate format (geographic coordinates)

Digital_Transfer_Option:

Online_Option:

Computer_Contact_Information:

Network_Address:

Network_Resource_Name: ftp://ftp.glerl.noaa.gov/

Access_Instructions: Connect to GLERLs anonymous ftp site - ftp.glerl.noaa.gov.  Move to the subdirectory ../gis/shoreline. Download files as desired based upon format  and geographic location.  README files located in each subdirectory explain directory and file naming conventions.

Online_Computer_and_Operating_System: Hewlett Packard - HP-UX 9.05

Offline_Option:

Offline_Media: CD-ROM

Recording_Capacity:

Recording_Density:

Recording_Density_Units:

Fees: to be determined

Ordering_Instructions: Contact the National Geophysical Data Center, NOAA

Turnaround:

Custom_Order_Process: None

Technical_Prerequisites: Able to use/convert SDTS, ARC/INFO, or ASCII-UNGENERATE files.

Available_Time_Period:

Beginning_Date/Time: 19971201

Ending_Date/Time: Undetermined

Metadata_Reference_Section:

Metadata_Date: 19971201

Metadata_Contact: Deborah H. Lee

Metadata_Standard_Name: FGDC Content Standards for Digital Geospatial Metadata

Metadata_Standard_Version:  19940608

Metadata_Time_Convention:  Local Time

Metadata_Security_Information:

Metadata_Security_Classification_System:  None

Metadata_Security_Classification:  Unclassified

Metadata_Security_Handling_Description:  None